Fischer Accuspin Micro 17/17R Microcentrifuge

Fisher Accuspin Micro 17/17R

Microcentrifuge

| Potential Hazards while performing these activities: |  |
| --- | --- |
| Electrical Hazard | This equipment contains electrical units that may come in contact with skin or liquids. |
| Slip Hazard | Because liquids are used with this unit, a slip hazard may be created. |
| Pinch Hazard | This equipment contains moving parts near the lid/agitator that may cause physical harm. |
| Chemical Hazard | Chemical spills or aerosolization may occur. |
| Fire Hazard | Any improper use of this equipment may cause a fire hazard. |

| Preventions to reduce exposure to hazards: |  |
| --- | --- |
| Eye Protection | Wear safety glasses while operating. |
| Protective Glove | Wear appropriate gloves for heat and chemical usage. |
| Non-Slip, Closed Shoes | Wear non-slip, closed shoes to avoid spills. |
| Read Manual | Become familiar with this guide before operating. |

| Note: If the rotor is not installed, contact the Lab Space Manager. |  |
| --- | --- |
| Setup Steps | Pictures |  |
| - Turn the Main Power switch ON. |  |  |
| - Allow unit to run initial check. When the speed and time readings show 0, you may proceed. |  |  |
| - Press the open lid button and verify the rotor is installed. |  |  |
| - Load samples into the rotor taking care to balance the load. |  |  |
| - Gently close the lid. |  |  |
| Operation | Pictures |  |
| - Use the RCF/RPM button to select the speed data input. |  |  |
| - Press and hold the rotation controls to adjust to desired speed. |  |  |
| - Use the time controls to set the run time. Holding the down arrow button will set continuous operation. |  |  |
| - Repeat step 3 to set the temperature on 17R models. The LED will flash when the pre-temp is reached. |  |  |
| - Press start to begin run. When time expires the instrument will stop. The stop button can be pressed any time to stop the rotor. |  |  |